Electric Power & Energy

- Draw a diagram to show a circuit that indicates a 90-V battery, an ammeter, and a resistance of 60 .

What does the ammeter read? 									(1.5 A)

- Draw a circuit diagram to include a 60-V battery, an ammeter, and a resistance of 12.5 . Indicate the ammeter reading. 											(4.8 A)

- Draw a circuit diagram to include a 16- resistor, a battery, and an ammeter that reads 1.75 A. Indicate the voltage of the battery. 										(28 V)

- A 15- electric heater operates on a 120-V outlet.

- What current flows through the heater? 							(8 A

)

- How much energy is used by the heater in 30 seconds? 					(2.9 x 104 J)

- How much heat is liberated by the heater in this time? 					(2.9 x 104 J)

- A 30- resistor is connected to a 60-V battery.

- What is the current in the circuit? 								(2 A)

- How much energy is used by the resistor in 5 minutes? 					(36000 J)

- The resistance of an electric stove element at operating temperature is 11 .

- 220 V are applied to it. What current flows through the element? 				(20 A)

- How much energy does the element use in 30 s? 						(1.3 x 105 J)

- The element is being used to heat a kettle containing 1.20 kg of water. Assume that 70 % of the heat

is absorbed by the water. What is its increase in temperature during the 30 s? 		(18 C)

- An electric heater is rated at 500 W.

- How much energy does the heater use in half an hour? 					(9.0 x 105 J)

- The heater is being used to heat a room containing 50.0 kg of air. If the specific heat of air is 1.1 kJ/kgC (1100 J/kgC) and 50% of the heat effectively heats the air in the room, what is its change in temperature? 										(8.2 C)

- A 100 W light bulb is 20% efficient

- How many joules does the light bulb convert into light each minute it is in operation? 	(1.2 x 103 J)

- How many joules of heat does the light bulb produce each minute? 			(4.8 x 103 J)

- How much energy does a 60 W light bulb use in half an hour?  If the light bulb is 25% efficient, how much heat does it generate during the half hour?  					(108,000 J ; 8.1 x 104 J)

- An electric space heater draws 15.0 A on a 120 V line.  If is operated, on the average, for 5.0 hours each day.

- How much power does the heater use? 							(1.8 kW)

- How much energy in kWh does it consume per month(30 days)? 				(270 kWh)

- At $0.08 per kWh, what does it cost to operate the heater per month? 			($21.60)

- A digital clock has an operating resistance of 4600  and is plugged into a 115 V outlet.

- How much current does it draw? 								(0.025A)

- How much power does it use? 								(2.9 W)

- If the owner of the clock pays $0.07 per kWh, what does it cost to operate the clock for a month (30 days)?  											(15 cents)